Sewing Machine Parts – YL 17-1

- Hand Wheel – manual control – moves take-up lever and needle up and down

- Take-Up Lever – feeds the thread from the spool to the needles

- Needle – feeds the thread that forms the upper part of the stitch

- Stitch Length Control (Regulator) – controls the length of the stitches

- Stitch Width Control – determines width of zigzag or decorative stitches

- Presser Foot – holds fabric in place against the feed dogs

- Feed Dogs – moves the fabric during stitching – found in the throat plate

- Throat Plate –seam allowance markings that help keep stitching straight

- Spool Pin – holds thread spools during sewing

- Bobbin – small spool that form the bottom part of the stitch

- Bobbin Winding Spindle – hold bobbin while thread is being wound on

- Reverse – button that allows the machine to stitch backwards

- Thread Tension Control – regulates tightness and looseness of the thread

- Foot Feed – pedal that controls starting, stopping and speed of the machine

- Bobbin Case – hold bobbin in place during sewing – under the throat plate

Young Living Chapter 18:1

- Using a Guide Sheet

  - A set of step-by-step instructions for sewing a pattern

  - How to use the pattern

  - A diagram of pieces

  - Explanation of pattern markings – guides for making a project

    - Cutting Line

    - Stitching Line

    - Grain Line

    - Dots

    - Notches

    - Place on Fold

    - Dart

  - Layout – diagram of how the pattern pieces should be placed on the fabric

- Preparing the Pattern Pieces

  - Cut apart pieces, but do not trim to exact measurement

  - Study each piece to check for markings

- Check Pattern Measurements

  - Check to see if the pattern fits your body type

  - Make length and width alterations before you cut

  - To lengthen – use parallel lines labeled “lengthen or shorten”

  - Width – measure pattern pieces to determine ease

  - Ease – the amount of fullness added to a pattern for movement and comfort

- Preparing the Fabric

  - Some fabric must be pre-washed to avoid shrinking

  - Check the grain of the fabric - The directions in which the treads run

    - Selvage – the tightly woven edge of the fabric that has no visible loose threads

    - Raw Edge – the unfinished edge of the fabric that has loose threads

    - Bias – the diagonal of the fabric

  - To see if fabric is on grain…

    - Fold fabric so the raw edges matched together

    - If the crosswise edges meet the fabric is on grain

Young Living Chapter 18:2

- Using Pattern pieces

  - Pinning

    - Fold your fabric so the right sides are together

    - Arrange pattern pieces on the fabric according to your layout

    - Lengthwise grain marking should be parallel to the selvage

    - Do not cut any pieces until they have all been pinned in place

Place pins diagonally to help keep fabric flat,   Place pins every 3 to 6 inches

  - Cutting

    - Place fabric flat on the table, one hand holds, the other cuts

    - Cut with long even strokes with the grain

    - Cut around the outside of the notches

    - Leave pattern pieces pinned to the fabric until you are ready to sew

    - Seam Allowance – fabric between seam line and raw edge

- Basic Sewing Skills

  - Seam

    - Lay two pieces of fabric placed together with the right sides facing each other.

    - Line up edges so they are even

    - Pin together and stitch guiding to the seam markings on throat plate

  - Stitching Straight Seams

    - Use the hand wheel to catch the bobbin thread and pull it through

    - Place fabric under the presser foot and line up to appropriate seam line

    - Backstitch – stitching over ½ inch of a seam at the beginning and the end to lock threads in place

    - Guide fabric with both hands, but DO NOT push

  - Stitching Curved Seams

    - Guide fabric with hands so the curves are smooth

    - Keep stitching and even distance from the edge

  - Turning Corners/Pivoting

    - When coming to a corner, turn the last few stitches using the hand wheel

    - Lift the presser foot, and leave the needle in the fabric

    - Turn the fabric to the correct position and lower presser foot

  - Seam Finished

    - Treatments used on the seam edges to prevent the fabric from raveling

    - Slight raveling – stitch ¼ inch from the edge,  Easy raveling – stitch a zigzag line

  - Casings

    - Fabric tunnels made to enclose elastic or drawstrings

    - Fold Down casing

      - Fold edge again to be a ½ inch wider than the elastic

      - Pin in place and stitch near edge

      - Leave a 2” opening to insert elastic

      - Insert elastic, stitch together, stitch opening closed